# 🚁 Autonomous Indoor Drone Navigation

## Installation Guide

This guide explains how to set up the complete simulation environment for the Autonomous Indoor Drone Navigation project using PX4 Autopilot, ROS 2 Humble, Gazebo Sim (Garden), and Micro XRCE-DDS Agent on Ubuntu 22.04 LTS.

## 📑 Table of Contents

- System Requirements

- Software Stack

- 1. Install Ubuntu 22.04 LTS

- 2. Install ROS 2 Humble

- 3. Install Colcon & ROS Tools

- 4. Install PX4 Autopilot v1.14

- 5. Install Gazebo Sim (Garden)

- 6. Install Micro XRCE-DDS Agent

- 7. Install PX4 ROS 2 Messages

- Useful Commands

- Troubleshooting

# System Requirements

| Component | Version |
| --- | --- |
| Operating System | Ubuntu 22.04.5 LTS (Jammy Jellyfish) |
| ROS Distribution | ROS 2 Humble Hawksbill |
| Flight Controller | PX4 Autopilot v1.14 |
| Simulator | Gazebo Sim (Garden) |
| Communication Bridge | Micro XRCE-DDS Agent v2.4.2 |
| Build System | Colcon + CMake |

# Software Stack

Gazebo Sim (Garden)
                           │
                Sensor Simulation
                           │
                           ▼
                 PX4 SITL (Flight Controller)
                           │
               XRCE-DDS (UDP Port 8888)
                           │
                           ▼
               Micro XRCE-DDS Agent
                           │
                    ROS 2 DDS Network
                           │
                           ▼
                ROS 2 Humble Workspace

# 1. Install Ubuntu 22.04 LTS

Verify your Ubuntu version:

lsb_release -a

Expected output:

Distributor ID: Ubuntu
Description: Ubuntu 22.04.5 LTS
Release: 22.04
Codename: jammy

Update the system:

sudo apt update
sudo apt upgrade -y

sudo apt install \
git \
curl \
wget \
build-essential \
cmake \
python3-pip \
python3-colcon-common-extensions \
-y

# 2. Install ROS 2 Humble

Install ROS 2 Humble by following the official ROS 2 installation guide.

Verify the installation:

echo $ROS_DISTRO

Expected output:

humble

# 3. Install Colcon & ROS Tools

sudo apt install \
python3-colcon-common-extensions \
python3-rosdep \
python3-vcstool \
-y

Initialize rosdep:

sudo rosdep init
rosdep update

# 4. Install PX4 Autopilot v1.14

Clone PX4:

cd ~

git clone -b v1.14.0 --recursive https://github.com/PX4/PX4-Autopilot.git

cd ~/PX4-Autopilot

Install dependencies:

bash ./Tools/setup/ubuntu.sh --no-sim-tools

Important

Reboot your system after the setup script completes.

Build PX4:

cd ~/PX4-Autopilot

make px4_sitl

Verify the version:

git describe --tags

Expected:

v1.14.0

# 5. Install Gazebo Sim (Garden)

Add the Gazebo repository:

sudo apt update

sudo apt install \
curl \
lsb-release \
gnupg \
-y

sudo wget https://packages.osrfoundation.org/gazebo.gpg \
-O /usr/share/keyrings/pkgs-osrf-archive-keyring.gpg

echo "deb [arch=$(dpkg --print-architecture) signed-by=/usr/share/keyrings/pkgs-osrf-archive-keyring.gpg] http://packages.osrfoundation.org/gazebo/ubuntu-stable $(lsb_release -cs) main" | sudo tee /etc/apt/sources.list.d/gazebo-stable.list

Install Gazebo:

sudo apt update

sudo apt install gz-garden

sudo apt install ros-humble-ros-gzgarden

Set the Gazebo version:

echo "export GZ_VERSION=garden" >> ~/.bashrc

source ~/.bashrc

Install additional ROS packages:

sudo apt install \
ros-humble-desktop \
ros-humble-ros-gzgarden-bridge \
ros-humble-octomap \
ros-humble-octomap-server \
ros-humble-octomap-msgs \
ros-humble-octomap-ros \
ros-humble-octomap-rviz-plugins \
ros-humble-pcl-ros

Verify:

gz sim --version

Expected:

Gazebo Sim, version 7.9.0

# 6. Install Micro XRCE-DDS Agent

Clone the repository:

git clone -b v2.4.2 https://github.com/eProsima/Micro-XRCE-DDS-Agent.git

cd Micro-XRCE-DDS-Agent

Build:

mkdir build

cd build

cmake ..

make

sudo make install

sudo ldconfig

Verify:

cd ~/Micro-XRCE-DDS-Agent

git describe --tags

Expected:

v2.4.2

# 7. Install PX4 ROS 2 Messages

Create a ROS workspace:

mkdir -p ~/px4_ros2_ws/src

cd ~/px4_ros2_ws/src

Clone the message package:

git clone -b release/1.14 https://github.com/PX4/px4_msgs.git

Build the workspace:

cd ~/px4_ros2_ws

source /opt/ros/humble/setup.bash

colcon build

Source the workspace:

source ~/px4_ros2_ws/install/setup.bash

To source automatically:

echo "source ~/px4_ros2_ws/install/setup.bash" >> ~/.bashrc

source ~/.bashrc

# Useful Commands

### Rebuild the ROS workspace

cd ~/px4_ros2_ws

colcon build --symlink-install

source install/setup.bash

### Kill all running simulation processes

killall -9 px4

killall -9 ruby

killall -9 MicroXRCEAgent

pkill -9 -f "gz sim"

pkill -9 -f "parameter_bridge"

# Troubleshooting

### MicroXRCEAgent: command not found

sudo make install

sudo ldconfig

### libmicroxrcedds_agent.so not found

sudo ldconfig

### fastcdr version 2 not found

Build the Micro XRCE-DDS Agent using the standalone CMake method instead of a ROS 2 colcon workspace to avoid dependency conflicts with ROS 2 Humble.

## ✅ Installation Complete

Your environment is now ready for:

- PX4 SITL

- Gazebo Sim (Garden)

- ROS 2 Humble

- Micro XRCE-DDS communication

- Offboard control

- Sensor integration

- Autonomous indoor drone navigation